## Yamaha Pacifica 604W

Top of Form

Made in:  Taiwan, Province of China Construction: Bolt-On

Orientation: Right handed Years: 1996 to 2003

Body Material: Alder

Body colour: Black, Cherry Sunburst, Transparent Blue

Body finish: Gloss : Pickguard colour: Pearloid White

Pickup Brand: Yamaha Pickup Configuration: HSS really SSSS

Tone knob: 1 Volume knob: 1 5 way switch Coil split:

Control colour: White

Description:

The Yamaha Pacifica 604W is an offset double-cut strat like guitar featuring the Wilkinson VS100  bridge made by Gotoh and Sperzel Trim-Lok locking tuners. It looks like this guitar has two single coil pickups with a humbucker in the bridge position. The bridge position actually contains two coupled single coil pickups making it a total of 4 Alnico V single coils in this guitar. Hence the 4 in the model name "Pacifica 604W". The tone knob doubles as a (push-push) coil tap.

Nut Width: 41 mm (1-5/8 inch) Thickness at nut: 20.90mm

Thickness at 12th fret: 22.90mm Neck Material: Maple

Neck colour: Natural Neck finish: Satin Fretboard: Rosewood

Fretboard radius: 350 mm (13.75 inch) Scale Length: 25.5 inch (648 mm)

Frets: 22 Fret Size: Medium Inlays: Dot Inlay colour: pearl

Locking tuners: Yes Headstock color:  Natural Headstock finish: Satin

Headstock angle: straight Tuning Machines: Sperzel Tuning Ratio: 12:1 Ratio

Bridge brand: Wilkinson Bridge type: Standard type

Hardware color: Chrome Satin Bridge: Wilkinson VS100